Sensitivity dependence of the planar Hall Effect sensor on the free layer of the spin-valve structure

Hung T.Q., Oh S.J., Tu B.D., Duc N.H., Phong L.V., AnandaKumar S., Jeong J.-R., Kim C.G.

Department of Materials Science and Engineering, Chungnam National University, Daejeon 305-764, South Korea; Department of Nano Magnetic Materials and Devices, College of Technology, Vietnam National University, Hanoi, Viet Nam

Abstract: Planar Hall effect (PHE) sensors with the junction size of 50 ? ?? 50 ? were fabricated successfully

by using spin-valve thin films Ta(5)/NiFe(x)/Cu(1.2)/NiFe(2)/IrMn(15)/Ta(5) (nm) with x = 4, 8,10,12,16. The magnetic field sensitivity of the PHE sensors increases with increasing thickness of ferromagnetic (FM) free layer. The sensitivity of about 95.5 m ?/(kA/m) can be obtained when the thickness of the FM-free layer increases up to 16 nm. The enhancement of sensitivity is explained by the shunt current from other layers. The PHE profiles are well described in terms of the Stoner-Wohlfarth energy model. The detection of magnetic micro-beads label Dynabeads?? M-280 is demonstrated and the results revealed that the sensor is
feasible for high-resolution biosensor applications. ?? 2009 IEEE.
Author Keywords: Biosensor applications; High field sensitivity; Micro-beads detection; Planar Hall effect

Index Keywords: Biosensor applications; Dynabeads; Energy model; Free layers; High field sensitivity; High resolution; Junction size; Magnetic field sensitivity; Micro-beads detection; Microbeads; Planar Hall effect; Shunt current; Spin-valve; Spin-valve structures; Stoner-Wohlfarth; Biosensors; Copper; Electric currents;  Gyrators;  Hall  effect;  Iridium  compounds;  Signal  transduction;  Spin  dynamics;  Tantalum; Magnetic  field  effects
Year: 2009

Source title: IEEE Transactions on Magnetics

Volume: 45

Issue: 6

Art. No.: 4957684

Page : 2374-2377

Cited by: 3

Link: Scorpus Link

Correspondence Address: Kim, C. G.; Department of Materials Science and Engineering, Chungnam
National University, Daejeon 305-764, South Korea; email: cgkim@cnu.ac.kr
ISSN: 189464

CODEN: IEMGA

DOI: 10.1109/TMAG.2009.2018578

Language of Original Document: English

Abbreviated Source Title: IEEE Transactions on Magnetics

Document Type: Conference Paper

Thank you for trying Solid Converter PDF Professional.

The trial version of this product only converts 10% of your document, with a 10 page maximum. 

For this conversion, Solid Converter PDF Professional converted 1 of 3 pages.

Please purchase Solid Converter PDF Professional at http://www.solidpdf.com/buy.htm to remove this restriction.
